# Design of a 4-Layer PCB Similar to Arduino Uno Using Atmega328p Microcontroller

# Assignment Requirements

This project involves designing a 4-layer PCB inspired by the Arduino Uno board architecture. The design integrates the Atmega328p microcontroller along with supporting circuitry, using Surface Mount Technology (SMT) components and proper PCB design techniques.

# Steps to Complete

## 1. Project Setup

• Create a new PCB design project in KiCad/Altium/Eagle (tool of your choice).
• Define a 4-layer stack-up:
   - Top Layer → Signal (high-speed digital traces, SMT components)
   - Inner Layer 1 → Ground plane (for noise reduction and stable reference)
   - Inner Layer 2 → Power plane (for distributing 5V/3.3V/AVCC)
   - Bottom Layer → Signal (slower digital and analog routing)

## 2. Schematic & Symbol/Footprint Design

• Draw the schematic for the Atmega328p (including reset, oscillator, power, and IO headers similar to Arduino Uno).
• Create custom symbols and footprints where necessary.
• Use SMT footprints for supporting components (resistors, capacitors, regulators, etc.).

## 3. PCB Layout

• Place the Atmega328p centrally, with headers aligned like Arduino Uno.
• Separate analog (ADC pins, AVCC) and digital (GPIO, UART, SPI, I2C) regions.
• Use ground and power planes for noise immunity.
• Route high-speed signals with controlled impedance.
• Add fiducials, mounting holes, and silkscreen labeling.

## 4. Verification & Outputs

• Run Design Rule Check (DRC) to eliminate errors.

Generate deliverables:
• PDF of schematic.
• Layer-wise PDF views of PCB layout.
• 3D view screenshots of the board.
• Screenshot showing layer stack-up and analog/digital separation.
• Gerber and Drill files for manufacturing.

# Project Description (Report Section)

This project replicates the design of an Arduino Uno using a 4-layer PCB with the Atmega328p microcontroller. By integrating schematic design, custom symbol/footprint creation, PCB layout, and manufacturing file generation, learners gain hands-on experience in real-world PCB design practices.

The 4-layer stack-up ensures proper power distribution and signal integrity, while careful layout separates analog and digital domains. Deliverables include schematic, layout, 3D views, and fabrication files — preparing learners for actual prototyping and PCB manufacturing.

Component analogy:

- MC33269D -> a low dropout voltage regulator (LDO)
It converts the input high voltage from the power jack to a stable lower voltage (5V or 3.3V)
- LMV358D -> dual low voltage op-amp, optimized for the 3.3V and 5V digital/analog systems, used for signal conditioning, best for interfacing the microcontrollers with the sensors.
It is used as: amplifier, filter, level shifter, differential amplifier, etcc.. (depends on the circuitry)
- The diode between the reset pin in the ATmega328p and the pd7 pin of the ATmega16U2 is used in the reset circuit to allow the 16U2 to trigger a reset on the 328P when uploading the code via the USB port, this is done while preventing reset signals. 
- Diode 1N4148 is used because it is a de facto component used since the 1970s, it is a small-signal switching diode that is designed for fast logical circuits. It is fast, cheap, reliable, low leakage, low capacitance. 
Other diodes are not used such as the Schottky diode (eg. BAT54), it could cause small reset instability or prevent proper reset release. Which is not ideal for signal isolation.
Rectifier diode (eg. 1N4001) ia designed for power and is way too slow for the digital logical signals, which could smear or delay the reset pulse.
- RN1D, (RN means resistor network), 1 is the label or the order of the resistor network, D means that we are addressing the 4th element (A, B, C, D)
- For the Arduino Uno schematic, I used the R_Pack04_Split with unit D
- For the ATmega328p circuit, I replaced the resonator with the crystal oscillator where we’ll see that the capacitors are not intrinsic in the design anymore but they are external capacitors with 22pF.
The crystal oscillators are faster and more accurate for the digital logical signals but they are bigger, more expensive (I am using it now because I might use ultra precise elements later, like the radio system or the GNSS)
-Polyfuse is used to protect the circuit from the overcurrent.
- The circular shape that is creuse in the middle or empty from the middle in the reset circuit is called SolderJUmper_2_opeen in KiCAD
-BLM21 is the ferrite bead that acts as the high frequency noise filter.
-LP2985-33DBVR is a low dropout linear voltage regulator made by Texas instrument, the input is 1 5V input from either the USB or from the VIN regulator and the output is a 3.3V output voltage for the external peripherals, it provides this 3.3V for sensors and shields.
Finally, it provides clean and low noise output.
-Another very important small component in the circuit is the MOSFET p-transistor called the FDN320P, it is used between the LMV358 and the LP2985-3.3 as an inverse current protector and as a signal switch for the same time. 
In simple terms, what happens in this power comparator circuitry is that it either gets powered by the computer input (USB) or the external VIN from the power jack barrel (5V) for both, what happens is that the op-amp LMV358, it acts as a comparator and orders the mosfet transistor whether to operate as on or off, if the external power supply is present, the comparator output disables the mosfet, if there is no external power supply, the mosfet is turned on.
-NCP1117-5.0-SOT223 is a power regulator that converts the higher level voltage to a lower level.
-For the decoupling capacitors, I use two types of capacitors, the polarized capacitors and the regular capacitors, the polarized capacitors (filled + and empty -) are used as decoupling and power filtering capacitors with typical values from 10 uF to 47 uF.
The second types of capacitors are the ceramic regular capacitors with typical values of 0.1uF to 1uF used as decoupling capacitors for filtering the high frequency noise components after the input stage.





Reviewed & Improved — Step 1: Requirements & Constraints

1) Electrical Requirements

Power Inputs

- USB-C (5 V input)

  - No USB-PD, so it operates in USB default mode at 5 V.

  - Use CC resistors (5.1 kΩ) to declare a UFP (device) and safely pull 5 V from VBUS.

- Barrel jack (2.1 mm center-positive, 7–12 V)

  - This input must feed a voltage regulator, normally an LDO or a buck converter.

  - Arduino Uno uses the NCP1117 (linear regulator).

  - If you use a linear regulator, note the heat dissipation constraints at higher input voltage.

Voltage Rails

- 5 V rail

  - Powered by USB-C or by the onboard regulator from the barrel jack.

  - Also used for ATmega328P I/O logic.

- 3.3 V rail

  - Needed for IMU, barometer, and other sensors.

  - Must be generated by a 3.3 V LDO with enough headroom (typ. 150–300 mA).

Logic Levels

- ATmega328P operates at 5 V

- Sensors operate at 3.3 V

  - Therefore:
I²C lines MUST be level-shifted
(Bidirectional MOSFET level shifter recommended).

Current Limits

- USB polyfuse (~500 mA hold, 1 A trip) protects the board from overcurrent.

- The ATmega328P does NOT handle 800 mA — this is incorrect.

  - ATmega328P max DC current is:

    - 40 mA per I/O pin

    - 200 mA TOTAL for all VCC + GND pins

So correct the statement to:

“The board includes a polyfuse that limits USB current to about 500 mA. The microcontroller can only source a total of about 200 mA across its pins.”

2) Interfaces

I²C Interfaces

- All sensors (IMU, barometer) communicate via I²C.

- The Grove connector will also expose SDA + SCL + 5V + GND.

- You must ensure:

  - Pull-up resistors are sized correctly (2.2k–10k depending on bus length).

  - The 3.3V I²C section is separated from the 5V I²C section using level shifting.

SPI / UART (Optional, but typically needed)

- Keep SPI free for:

  - Programming (ISP)

  - Possible future expansions

- The ATmega16U2 uses UART to communicate with the ATmega328P (USB-to-serial).

3) Programming Interfaces

ISP Headers

You need TWO separate 2×3 ISP headers:

✔ ISP #1 — ATmega16U2

- Used to flash the USB-serial firmware
(same as Arduino Uno’s “16U2 USB CDC firmware”)

✔ ISP #2 — ATmega328P

- Used to:

  - Burn the bootloader

  - Program fuses

  - Recover a bricked MCU

  - Upload sketches if USB is not working

2×2 Header

- A 2×2 header is not used for analog.

- Arduino does not use 2×2 headers for analog signals.

Common possible uses:

- SWD/JTAG for other chips

- Debug output

- Secondary power/debug connector

- I²C breakout

- “FTDI header style” serial pins (not common on Uno-style boards)

Clarify the exact purpose before including it.

4) Mechanical Properties

Board Dimensions

- Length: 68.6 mm

- Width: 53.4 mm
(This matches the standard Arduino Uno R3 PCB outline.)

Mounting

- Four mounting holes in Arduino Uno standard pattern.

Connector Placement

- USB-C and barrel jack on the top edge.

- ISP headers near their respective microcontrollers.

- Grove connector near one board edge for easy access.

Sensor Placement

- IMU + barometer should be:

  - As close to the geometric center of the board as possible
(for stable readings and reduced vibration bias)

  - On a section with minimal copper under them (IMU requirement)

  - Physically separated from high-noise components (crystal, switching regulators)

- Adding custom interfaces:

1) The first thing I did was to replace the USB-B with the USB-C to adapt to the most modern devices that always use USB-C plug type.
Adding USB-C connector just required adding the pull down resistors of 5.1kohms.

2) The second custom interface is adding the grove connector, which is a plug and play hardware connector (the white thing with pins) used in modular electronics.
Grove modules can be grouped into 2 groups:
a- Sensors as inputs
b- Actuators as outputs
Grove connector is typically a 4-pin connector, with signals as follows:
a- VCC
b- GND
c- SIG1 (SCL)
d- SIG2 (SDA)